## 1. Executive Summary & Context

As a quant on an energy trading desk in France, you are tasked with managing a Virtual Power Plant (VPP). Your asset consists of a aggregate cluster of distributed wind generation, solar farms, run of river and residential heating/cooling loads, paired with a utility-scale Battery Energy Storage System (BESS).

Your mandate is pure physical baselining and load-shaping. You will fetch historical French grid data from ODRÉ (Open Data Réseaux Énergies), extract weather-dependent generation and demand signals, establish a baseline load, and write a mathematical program in Python to optimize battery operations to transform an unpredictable, weather-driven portfolio into a flat Baseload (BL) delivery profile.

## 2. Data Sourcing & API Endpoint

You will fetch historical national electricity grid data directly from the RTE éCO2mix public API provided by ODRÉ.

- Dataset Web Interface: ODRÉ éCO2mix National Consolidated Data

- API Base URL:

https://odre.opendatasoft.com/explore/dataset/eco2mix-national-cons-def/information/?disjunctive.nature

https://odre.opendatasoft.com/api/explore/v2.1/catalog/datasets/eco2mix-national-cons-def/records?limit=20

### Required API Fields to Extract:

- date_heure: Timestamp (ISO 8601 format)

- consommation: Total National Consumption (MW)

- eolien: Onshore/Offshore Wind Power Generation (MW)

- solaire: Solar PV Power Generation (MW)

- Nucleaire: Nucléaire (MW)

- Run of river: Hydraulique - Fil de l'eau + éclusée (MW)

## 3. Physical Asset Specifications

You are provided with a utility-scale 4-hour Lithium-ion Battery System with the following physical parameters:

| Parameter | Notation | Value | Notes |
| --- | --- | --- | --- |
| Max Charge/Discharge Power | P_max | 5MW | Maximum rate of power exchange |
| Nameplate Energy Capacity | E_max | 20 MWh | Total storage size (4-hour duration) |
| Usable Storage Bounds | SoC_min, SoC_max | 10%, 90% | Operating limits 2 MWh to 18 MWh |
| Round-Trip Efficiency (RTE) | eta | 85% | One-way efficiency eta_one-way = sqrt(0.85) = 0.922 |
| Initial State of Charge | SoC_0 | 50% (10 MWh) | State of charge at t=0 |

## 4. Step-by-Step Assignment Instructions

### Step 1: Programmatic Data Ingestion & Cleaning

- Write a Python function using requests or httpx to query the ODRÉ API for 2020-2025

- Parse the JSON response into a pandas DataFrame.

- Turn data into hourly

### Step 2: VPP Portfolio Net Load Calculation

Define the raw, unhedged net demand profile of your VPP asset portfolio at each hour (t):

NetLoad_t = Consommation_t - (Eolien_t + Solaire_t  + +Nucleaire_t +  Ror_t)

Calculate the target Baseload (𝐿), defined as the rolling or global mean of the net load across the evaluation horizon:

𝐿=1𝑇𝑡=1𝑇NetLoadt

Realistically you won't have more than 5% of France's power portfolio so you can multiply this value by 5% and your NetLoad as of this point by 5%

If you deploy a single1 MW/4 MWhery against this net load, what percentage of the profile variance will it eliminate? Will it make a visible difference? Prove it with a quick calculation.

How many batteries do we need? If one is not enough?

### Step 3: Mathematical Formulation of the Optimization Model

You must formulate the dispatch problem as a Linear Program (LP) or Quadratic Program (QP) using Python optimization packages (CVXPY, PuLP, or SciPy.optimize).

For each hour 𝑡∈1,2,…,𝑇 :

- Let 𝑝𝑡charge≥0 be the power drawn from the grid to charge the battery.

- Let 𝑝𝑡discharge≥0 be the power injected into the grid from the battery.

#### Objective Function:

Minimize the squared deviation of the VPP's net profile from the Baseload target (flattening the profile):

What does this mean ? What am I trying to do with my batteries ?

#### System & Physics Constraints:

- Power Limits:

0≤𝑝charge𝑡≤𝑃𝑚𝑎𝑥, ∀𝑡

0≤𝑝discharge𝑡≤𝑃𝑚𝑎𝑥, ∀𝑡

- State of Charge (SoC) Accounting:

𝑆𝑜𝐶𝑡=𝑆𝑜𝐶𝑡−1+𝑝charge𝑡⋅𝜂one-way−𝑝discharge𝑡𝜂one-way⋅Δ𝑡, ∀𝑡≥1

- Storage Boundary Constraints:

𝑆𝑜𝐶𝑚𝑖𝑛⋅𝐸𝑚𝑎𝑥≤𝑆𝑜𝐶𝑡≤𝑆𝑜𝐶𝑚𝑎𝑥⋅𝐸𝑚𝑎𝑥, ∀𝑡

- End-of-Horizon Boundary Condition (Cyclic Neutrality):

𝑆𝑜𝐶𝑇=𝑆𝑜𝐶0=10 MWh

## 5. Required Deliverables & Visualizations

In your final report/notebook, you must include:

- Mathematical Documentation: Clearly state your decision variables, objective function, and constraints.

- Visualizations (Plotly or Matplotlib):

- Chart 1: Unadjusted VPP Net Load vs. Adjusted VPP Net Load (post-BESS dispatch) vs. Baseload Target Line (𝐿).

- Chart 2: BESS State of Charge (𝑆𝑜𝐶𝑡) trajectory over time alongside Charge/Discharge power streams.

- Quantitative Performance Analysis:

- Compute the Variance Reduction Factor (%) of the Net Load profile post-optimization.

- Calculate the total energy loss (MWh) incurred due to round-trip efficiency losses (𝜂= 0.85 ).